Development and Multi-objective Calibration
of a Raven HBV-EC Model for the
Grand River below Shand Dam

Name- Akash Patel

Contents

1	Introduction	4

2	Study Watershed and Data Sources	4

2.1	Study Area	4

2.2	Input Data, Data Quality, Sources, and Pre-processing	6

3	Subbasin and HRU Discretization	7

4	Conceptual Model and Selected Algorithms	8

4.1	Selection of the HBV-EC Model	8

4.2	Conceptual Storage and Flow Structure	9

4.3	Selected Raven Algorithms	10

4.4	Routing and Reservoir Representation	12

5	Model Calibration Approach and Results	13

5.1	Calibration Objectives and Diagnostic Metrics	13

5.2	Baseline Simulation	14

5.3	Manual Calibration	14

5.4	Automated PADDS Calibration	16

5.5	Final Calibration Results	17

6	Results and Discussion	18

6.1	Calibration progress and final diagnostics	18

6.2	Multi-objective behaviour and parameter interpretation	19

6.3	Model limitations and uncertainty	20

7	Answer to the Model Question	21

7.1	Answer to the project question	21

8	Conclusions	22

9	Reference	23

Table of Figures

Figure 2-1 Regional location and final ArcGIS/BasinMaker watershed discretization, showing the watershed boundary, 18 subbasins, stream network, outlet gauge, and four HRU land-use classes.	6

Figure 6-1 Observed and simulated daily hydrograph for the selected PADDS solution at WSC 02GA016.	19

Table of Tables

Table 2-1 . Input data, source and preprocessing	7

Table 3-1 Final HRU composition from the Raven .rvh file.	8

Table 4-1 Active Raven algorithms and their role in the model.	10

Table 5-1 Baseline result	14

Table 5-2 Model Parameters for Calibration	14

Table 5-3 Manual Calibration Result	15

Table 5-4 PADDS parameter bounds and selected final compromise values.	16

Table 5-5 Final Result	17

# Introduction

Hydrologic models use meteorological data and landscape properties to estimates of snow storage, evapotranspiration, soil moisture, groundwater contribution, and streamflow. Their value depends not only on statistical fit, but also on whether the selected structure represents the processes that control the observation being simulated. This distinction is especially important for a gauge located directly below a regulated reservoir, where measured discharge reflects both watershed inflow and human operating decisions.

The Grand River basin upstream of Shand Dam was selected because it contains cold-season snow processes, extensive agricultural land, lakes, and a regulated outlet.

# Study Watershed and Data Sources

## Study Area

The model domain drains southward to WSC station 02GA016, located on the Grand River below Shand Dam in southern Ontario, as shown in Figure 2-1. The watershed was delineated using GIS-based terrain, drainage-network, subbasin, and land-use information. The original GIS-delineated watershed area was approximately 787.29 km². After the BasinMaker, spatial data were divided into hydrological response units and transferred to Raven, the HRU areas summed to 780.01 km². Therefore, 780.01 km² was used as the effective watershed area represented by the hydrologic model. The difference of approximately 7.28 km², or 1%, likely resulted from polygon clipping, the removal or merging of very small HRU polygons, unclassified spatial areas, and small gaps introduced during HRU processing. This difference represents a minor source of spatial uncertainty but is sufficiently small that it is not expected to substantially affect the overall basin water balance.

The watershed contains several subbasins connected by the Grand River and its tributary stream network. Agricultural land is the dominant land-use class, with smaller areas of forest, urban development, lakes, and other surface-water features. Because of the extensive agricultural coverage, the watershed response is strongly influenced by soil-moisture storage, infiltration, evapotranspiration, shallow groundwater movement, and potentially artificial agricultural drainage. Forested and lake areas provide additional interception and storage, while developed areas can produce a faster runoff response. The area-weighted mean HRU elevation was approximately 477 m, with HRU centroid elevations ranging from approximately 398 to 510 m. The area-weighted mean slope was approximately 3.61°, indicating that much of the basin has relatively gentle terrain.

The climate of the watershed produces important seasonal variations in runoff. During winter, precipitation may be stored temporarily as snow, followed by increased runoff during spring snowmelt. Consequently, the representation of precipitation phase, snow accumulation, snowmelt, soil-water storage, evapotranspiration, percolation, and groundwater release is important for reproducing the seasonal hydrograph. Lakes and other surface-water feature also influence runoff storage and timing.

The available hydrometeorological data included daily precipitation, daily minimum and maximum air temperature, and daily observed discharge at WSC station 02GA016. Monthly potential evapotranspiration and monthly average temperature values were also used to support the selected Raven evapotranspiration method. The spatial datasets included the watershed boundary, subbasin boundaries, stream network, digital elevation information, land-use classification, lakes, HRU polygons, HRU elevations, slopes, and contributing areas. These datasets were processed into the Raven watershed, parameter, meteorological, and model-configuration files required for simulation.

An important limitation is that station 02GA016 is located below Shand Dam. Therefore, the measured discharge represents a regulated dam release rather than completely natural watershed runoff. Reservoir storage and operational decisions can change the magnitude and timing of downstream flow, particularly during high-flow events, low-flow periods, and seasonal water-management operations. Because detailed reservoir-operation rules were not explicitly represented in the model, differences between simulated and observed discharge may reflect both hydrologic-model uncertainty and the influence of dam regulation.

Figure 2-1 Regional location and final ArcGIS/BasinMaker watershed discretization, showing the watershed boundary, 18 subbasins, stream network, outlet gauge, and four HRU land-use classes.

## Input Data, Data Quality, Sources, and Pre-processing

The model required daily streamflow and meteorological time series together with spatial data describing topography, land use, and soils. The main datasets and processing steps are summarized in Table 2-1.

Table 2-1 . Input data, source and preprocessing

| Input dataset | Source | Main preprocessing |
| --- | --- | --- |
| Daily streamflow | Environment and Climate Change Canada, WSC station 02GA016 | Data were downloaded using a script developed for the project. The downloaded file contained more than one parameter; Parameter 1, representing daily discharge in m³/s, was selected. Dates, missing values, duplicates, and units were checked, and the processed record was converted to the Raven Qobs_daily input file. |
| Daily meteorological data | Environment and Climate Change Canada climate gauges | Daily maximum temperature, minimum temperature, rainfall, and snowfall were obtained from gauges within or near the watershed. Missing observations were replaced using corresponding data from nearby gauges; where several neighbouring records were available, their average was used. |
| Monthly PET and temperature | Calculated from the processed meteorological record | Potential evapotranspiration was calculated from the available climate data. Twelve monthly PET values and twelve-monthly average-temperature values were then prepared for Raven’s PET_FROMMONTHLY method. |
| Digital elevation model | HydroSHEDS | The DEM was downloaded and processed for BasinMaker. It was clipped to the study area and used to delineate drainage features, subbasins, and watershed topography in BasinMaker. An elevation-band layer was created in ArcGIS Pro for assigning representative elevation information to the model units. |
| Land cover | 2020 Land Cover of Canada dataset from the Government of Canada Open Data portal | The land-cover data were clipped to the watershed and create shapefile for BasinMaker |
| Soil data | Ontario GeoHub | Soil information was downloaded, clipped to the watershed and create shapefile for BasinMaker |

# Subbasin and HRU Discretization

The final BasinMaker-derived .rvh file contains 18 hydrologically connected subbasins. Connectivity terminates at subbasin 90014270, the gauged outlet at 02GA016. The network also includes nested gauged locations such as 02GA041, 02GA022, 02GA027, and 02GA001. Eighteen subbasins were retained because they preserve the main drainage topology and lake locations while remaining manageable for a course-scale calibration exercise. Catchment routing operates within this network, even though explicit channel routing was disabled in the final .rvi configuration.

Within the subbasins, the watershed was divided into 44 hydrological response units. An HRU represents an area that is assigned similar land-surface and hydrological properties. The final discretization was based mainly on the intersection of subbasin boundaries with four simplified land-use classes: agriculture, forest, lake, and urban land.

Table 3-1 Final HRU composition from the Raven .rvh file.

| HRU type | Number of HRUs | Area (km2) | Area (%) |
| --- | --- | --- | --- |
| Agriculture | 23 | 645.869 | 82.80 |
| Forest | 17 | 115.358 | 14.79 |
| Lake | 2 | 16.998 | 2.18 |
| Urban | 2 | 1.787 | 0.23 |
| Total | 44 | 780.012 | 100.00% |

A limitation is that the HRUs were differentiated mainly by land use, while the major non-lake HRUs used the same general soil profile. Therefore, spatial variations in soil texture, drainage class, wetlands, groundwater connectivity, and agricultural tile drainage were not explicitly represented. These simplifications may limit the model’s ability to reproduce local runoff generation and sustained baseflow.

# Conceptual Model and Selected Algorithms

## Selection of the HBV-EC Model

The watershed was simulated using the HBV-EC conceptual hydrologic model within the Raven Hydrological Modelling Framework. HBV-EC was selected because it represents the principal hydrological processes expected in a snow-affected, mixed-land-use watershed while retaining a manageable number of parameters. The model includes rain–snow partitioning, snow accumulation and melt, canopy interception, evapotranspiration, soil-water storage, nonlinear runoff generation, fast and slow runoff responses, percolation, catchment routing, and reservoir storage.

This process structure is suitable for the Grand River watershed because winter precipitation can be temporarily stored as snow and released during spring melt. Agricultural land dominates the watershed, making soil-moisture storage, evapotranspiration, infiltration, and shallow subsurface drainage important controls on runoff. The observed hydrograph also contains both rapid event responses and sustained lower flows, requiring separate fast and slow storage components. Lakes and Shand Dam further modify the timing and magnitude of downstream discharge.

HBV-EC was preferred over a more complex physically based model because the available calibration data were mainly daily meteorological observations and discharge at one downstream gauge. A more detailed model would require additional measurements of soil properties, groundwater levels, channel geometry, snow conditions, and reservoir operations. The HBV-EC structure therefore provided an appropriate balance between hydrological representation, parameter identifiability, available data, and computational efficiency. Glacier processes from the original course template were removed because the watershed contains no glacier HRUs.

## Conceptual Storage and Flow Structure

The model used a three-layer conceptual structure consisting of TOPSOIL, FAST_RES, and SLOW_RES storages. Precipitation and snowmelt first become available at the land surface. After interception and evaporation losses, water enters the topsoil layer, where it may be retained as soil moisture or converted to runoff depending on the wetness of the soil.

The HBV infiltration formulation produces relatively little runoff when the soil is dry because a larger proportion of incoming water is retained in storage. As the soil approaches saturation, a greater proportion becomes runoff. This behaviour is controlled mainly by topsoil porosity and the HBV_BETA parameter.

Runoff generated from the topsoil is transferred to the fast reservoir. This reservoir represents rapid pathways such as event runoff, shallow subsurface flow, and drainage responses. Water is also transferred from the fast reservoir to the slow reservoir through percolation. The slow reservoir represents delayed groundwater storage and supplies sustained baseflow during periods without rainfall or snowmelt.

The complete conceptual sequence was:

Meteorological forcing → rain–snow partitioning → snow accumulation and melt → canopy interception → topsoil storage and evapotranspiration → fast reservoir → slow reservoir → catchment routing → reservoir routing → outlet discharge.

This structure was selected because one storage reservoir would not adequately represent both rapid event peaks and long-duration groundwater recession. The separate fast and slow reservoirs allow peak flow, hydrograph recession, and low-flow persistence to be adjusted independently, although interactions among their parameters remain important.

## Selected Raven Algorithms

The final simulation used a daily time step and the ORDERED_SERIES numerical solution method. The selected algorithms and their model roles are summarized below.

Table 4-1 Active Raven algorithms and their role in the model.

| Hydrological component | Raven algorithm | Role and justification |
| --- | --- | --- |
| Numerical solution | ORDERED_SERIES | Applies the hydrological processes sequentially during each daily time step. |
| Orographic correction | OROCORR_HBV | Adjusts precipitation, temperature, and evapotranspiration for elevation differences between the climate gauge and HRUs. |
| Rain–snow partitioning | RAINSNOW_HBV | Uses air temperature to divide precipitation into rainfall and snowfall. |
| Potential snowmelt | POTMELT_HBV | Estimates melt using a temperature-index or degree-day approach. |
| Snow balance | SNOBAL_SIMPLE_MELT | Removes meltwater from the snowpack and transfers it to available surface water. |
| Potential evapotranspiration | PET_FROMMONTHLY | Uses 12 monthly evaporation values to represent the seasonal pattern of atmospheric demand. |
| Infiltration and runoff generation | INF_HBV | Controls runoff generation according to relative soil wetness and HBV_BETA. |
| Soil evaporation | SOILEVAP_HBV | Limits actual soil evaporation according to potential evaporation and available soil water. |
| Capillary rise | RISE_HBV | Transfers water upward from subsurface storage toward the topsoil under dry conditions. |
| Percolation | PERC_CONSTANT | Transfers water from fast storage to slow groundwater storage. |
| Fast runoff | BASE_POWER_LAW | Produces nonlinear rapid runoff and controls event peaks and short recession. |
| Slow runoff | BASE_LINEAR | Produces delayed groundwater discharge and sustained baseflow. |
| Catchment routing | ROUTE_TRI_CONVOLUTION | Delays and attenuates runoff moving from HRUs to subbasin outlets. |
| Channel routing | ROUTE_NONE | No separate hydraulic routing algorithm was used for individual channel reaches. |
| Reservoir routing | RESROUTE_STANDARD | Provides simplified storage–outflow routing for two modelled lakes or reservoirs. |

The rain–snow and melt algorithms were selected because daily temperature data were available, while the solar radiation, humidity, wind, and snow-energy observations required for a full energy-balance snow model were not consistently available. The degree-day approach is simpler but remains appropriate for a daily conceptual model in southern Ontario.

PET_FROMMONTHLY was selected to reproduce the seasonal pattern of evapotranspiration using 12 monthly values. This approach captures low winter evaporation and higher summer demand but cannot reproduce individual hot, cloudy, humid, or windy days. Consequently, daily and interannual variations in evapotranspiration are simplified.

INF_HBV was chosen because runoff generation in the agricultural watershed depends strongly on antecedent soil moisture. The nonlinear formulation allows more precipitation to be stored when the soil is dry and produces a greater runoff fraction when the soil is wet. This behaviour is hydrologically reasonable for the basin, although actual processes such as tile drainage, preferential flow, and wetland connectivity are represented only indirectly through calibrated parameters.

## Routing and Reservoir Representation

Runoff generated by the HRUs was routed within each subbasin using ROUTE_TRI_CONVOLUTION. This method provides a simplified travel-time distribution that delays and attenuates runoff before it reaches the subbasin outlet. It was selected because detailed cross-sections, roughness coefficients, and reach-specific travel-time observations were unavailable for the complete network.

Explicit channel routing was set to ROUTE_NONE. Therefore, the model does not simulate detailed hydraulic movement through each stream reach. This simplification reduces the number of poorly constrained routing parameters but limits the model’s ability to reproduce exact peak timing and attenuation.

Two lake or reservoir features were represented using RESROUTE_STANDARD. The outlet reservoir was described using BasinMaker-derived geometry and a generic weir relationship. The modelled outlet lake had an approximate area of 5.55 km², maximum depth of 11 m, crest width of 46.55 m, and generic weir coefficient of 0.6. This representation allows inflow to be stored and attenuated before release, but it does not reproduce actual Shand Dam gate settings, seasonal target levels, operating rule curves, flood-management decisions, or minimum-flow releases.

The simplified reservoir representation is the most important structural limitation of the configuration. WSC station 02GA016 records regulated outflow below the dam, while the Raven model calculates release primarily from modelled storage and a generic outlet relationship. Therefore, even a reasonable simulation of natural watershed inflow may not reproduce the observed daily discharge.

# Model Calibration Approach and Results

## Calibration Objectives and Diagnostic Metrics

Calibration was completed to improve agreement between simulated and observed daily discharge at WSC station 02GA016. Four diagnostic metrics were used because no single statistic adequately describes all aspects of hydrograph performance.

Nash–Sutcliffe Efficiency (NSE) measures how well the simulated daily hydrograph reproduces observed variation relative to the observed mean. Because errors are squared, NSE is strongly influenced by large flows and incorrectly simulated peaks.

Root-mean-square error (RMSE) reports the typical error magnitude in the same units as discharge, m³/s. Like NSE, it gives greater weight to large errors.

Kling–Gupta Efficiency (KGE) evaluates a combination of correlation, mean bias, and relative variability. It provides a more balanced measure of timing, average flow, and hydrograph variability than NSE alone.

Percent bias (PBIAS) evaluates the difference between total simulated and observed discharge volume. Using the sign convention applied in this project, negative PBIAS indicated overall underestimation of observed discharge. Absolute PBIAS was used during optimization because both overestimation and underestimation are undesirable.

The use of four metrics was important because a parameter set could achieve a better NSE by matching a few large peaks while worsening total volume or low-flow behaviour. The selected model was therefore required to provide a reasonable compromise among peak-sensitive performance, total error, variability, and water balance.

## Baseline Simulation

The baseline simulation was completed using the initial HBV-EC parameter values before calibration. It produced:

Table 5-1 Baseline result

| Metric | NSE | RMSE | KGE | PBIAS |
| --- | --- | --- | --- | --- |
|  | 0.2355 | 14.154 | 0.5149 | −26.85% |

The baseline NSE and KGE indicated limited agreement between the observed and simulated daily hydrographs. The PBIAS showed substantial overall underestimation of discharge volume. Hydrograph inspection also indicated that the model did not reproduce the magnitude and timing of several observed events.

The baseline run is very important. It gives a reference against which all other later calibration improvements could be measured. It also helped to identify whether the main problem was associated with the total water balance, event response, snowmelt timing, or recession behaviour.

## Manual Calibration

Manual calibration was completed before automated calibration. Six influential parameters were selected:

Table 5-2 Model Parameters for Calibration

| Parameter | Main hydrological effect |
| --- | --- |
| TOPSOIL POROSITY | Effective soil-water storage and total runoff volume |
| HBV_BETA | Nonlinearity of runoff generation under dry and wet conditions |
| FAST_RES BASEFLOW_COEFF | Rapid runoff release, peak magnitude, and short recession |
| FAST_RES MAX_PERC_RATE | Transfer from fast storage to slow groundwater storage |
| MELT_FACTOR | Snowmelt rate, spring peak magnitude, and timing |
| SLOW_RES BASEFLOW_COEFF | Sustained groundwater contribution and low-flow recession |

One parameter was initially changed at a time and observe the result. After each Raven run, all four metrics and the hydrograph were reviewed. The last best value was retained before testing another parameter. This procedure allowed the effect of each parameter to be interpreted rather than changing all parameters simultaneously without understanding their influence.

Topsoil porosity and HBV_BETA were examined first because they control soil storage and the proportion of incoming water converted to runoff. The fast-reservoir coefficient was then adjusted to examine event peaks and recession. Maximum percolation and the slow-reservoir coefficient were used to alter the transfer and persistence of groundwater flow. The melt factor was considered after the broader water-balance and runoff-partitioning behaviour had been assessed.

The selected manual simulation produced:

Table 5-3 Manual Calibration Result

| Metric | NSE | RMSE | KGE | PBIAS |
| --- | --- | --- | --- | --- |
| Baseline | 0.2355 | 14.154 | 0.5149 | −26.85% |
| Selected manual run | 0.2244 | 14.257 | 0.5972 | −13.60% |

Manual calibration improved KGE by approximately 0.0823 and reduced the magnitude of PBIAS by approximately 13.25 percentage points. Therefore, the overall water balance, mean flow, and composite hydrograph behaviour improved substantially. However, NSE decreased slightly and RMSE increased slightly. This means that the manual parameter set did not improve the squared daily errors, even though it improved the average volume and KGE.

## Automated PADDS Calibration

Automated calibration was completed using OSTRICH version 17.12.19 and the Pareto Archived Dynamically Dimensioned Search (PADDS) algorithm. PADDS was chosen because the calibration problem has four competing objectives rather than one objective.

The Raven parameter file was converted to an OSTRICH template containing the six decision variables. For each model evaluation, OSTRICH inserted a new parameter set, executed Raven, and extracted NSE, RMSE, and KGE from Diagnostics.csv. A separate PowerShell procedure calculated absolute PBIAS. Because OSTRICH performs minimization, NSE and KGE were multiplied by −1 internally, while RMSE and absolute PBIAS were minimized directly.

The PADDS settings included:

- 200 model evaluations;

- random seed 111;

- perturbation value 0.20;

- random selection from the Pareto archive;

- four simultaneous objectives.

The selected parameter bounds and final values were:

Table 5-4 PADDS parameter bounds and selected final compromise values.

| Parameter | Lower bound | Upper bound | Selected Run 138 |
| --- | --- | --- | --- |
| TOPSOIL POROSITY | 0.05 | 0.30 | 0.050000 |
| HBV_BETA | 0.50 | 7.00 | 0.519863 |
| FAST_RES BASEFLOW_COEFF (d⁻¹) | 0.02 | 0.80 | 0.769155 |
| MAX_PERC_RATE (mm/d) | 5 | 50 | 48.7398 |
| MELT_FACTOR (mm/d/°C) | 2 | 10 | 6.94394 |
| SLOW_RES BASEFLOW_COEFF (d⁻¹) | 0.001 | 0.20 | 0.106550 |

PADDS generated 24 non-dominated solutions in the final Pareto archive. A solution was non-dominated when no other solution improved one objective without worsening at least one of the remaining objectives. Therefore, the archive represented a range of possible compromises rather than one universally best parameter set.

To select a reproducible final solution, the four objectives were normalized between the best and worst values in the Pareto archive. Equal weight was assigned to NSE, RMSE, KGE, and absolute PBIAS, and the Euclidean distance from each solution to the ideal objective vector was calculated. Run 138 had the smallest normalized distance and was selected as the final balanced solution. Raven was then run independently using the exact Run 138 parameter values to confirm the final diagnostics and hydrograph.

The multi-objective approach prevented selection of a misleading solution based only on NSE. For example, another evaluated parameter set achieved NSE of approximately 0.312 and RMSE of approximately 13.43 m³/s, but its KGE decreased to approximately 0.492 and its absolute PBIAS increased to approximately 34.55%. Although it performed better for the peak-sensitive metrics, its overall flow volume and composite hydrograph behaviour were much worse. Run 138 was therefore more suitable because it remained reasonably close to the best values for all four objectives.

## Final Calibration Results

The results are summarized below.

Table 5-5 Final Result

| Model stage | NSE | RMSE (m³/s) | KGE | PBIAS (%) |
| --- | --- | --- | --- | --- |
| Baseline | 0.2355 | 14.154 | 0.5149 | −26.85 |
| Selected manual run | 0.2244 | 14.257 | 0.5972 | −13.60 |
| Selected PADDS Run 138 | 0.2879 | 13.661 | 0.6370 | −10.00 |

Compared with the baseline, the final PADDS solution increased NSE by approximately 0.052, reduced RMSE by approximately 0.49 m³/s, increased KGE by approximately 0.122, and reduced absolute PBIAS by approximately 16.85 percentage points. It also improved all four metrics relative to the manual run. Automated calibration therefore produced a genuine overall improvement rather than improving one statistic by sacrificing the others.

However, the final metrics indicate only moderate performance. The positive but relatively low NSE shows that considerable daily variability remained unexplained. KGE improved but remained well below a near-perfect value. The remaining negative PBIAS indicates that total simulated discharge was still approximately 10% lower than observed discharge.

# Results and Discussion

## Calibration progress and final diagnostics

The final PADDS solution improved all four diagnostics compared with the baseline. NSE increased by 0.052, RMSE decreased by 0.49 m3/s, KGE increased by 0.122, and absolute PBIAS decreased by 16.85 percentage points. Relative to the selected manual run, PADDS also increased NSE and KGE, decreased RMSE, and further reduced the volume error. The automatic calibration therefore provided a defensible improvement rather than merely shifting performance from one metric to another.

Figure 6-1 Observed and simulated daily hydrograph for the selected PADDS solution at WSC 02GA016.

The hydrograph shows that the model reproduces the occurrence of several spring and rainfall-driven responses, but peak magnitude and timing remain inconsistent. The largest observed event in the output was approximately 225 m3/s on April 16, 2014; the simulated flow on that date was approximately 120 m3/s, while the simulated maximum of approximately 164 m3/s occurred three days earlier. The flow-duration curve shows a second limitation: simulated moderate and low flows decline much more rapidly than observed flows. Observed median discharge was approximately 5.18 m3/s, whereas the simulated median was approximately 1.95 m3/s. This sustained low-flow mismatch is consistent with reservoir releases and other basin storage processes that are not fully represented by the conceptual structure.

## Multi-objective behaviour and parameter interpretation

Several runs achieved higher NSE and lower RMSE by accepting much poorer KGE or volume bias. For example, the last evaluated run produced NSE = 0.312 and RMSE = 13.43 m3/s, but KGE fell to 0.492 and absolute PBIAS increased to 34.55%. Run 138 was selected because it remained close to the best archive values across all four objectives rather than maximizing only NSE.

Three selected parameters approached a search bound: POROSITY reached its lower bound, HBV_BETA was close to its lower bound, and the fast reservoir baseflow coefficient was close to its upper bound. This boundary-seeking behaviour suggests that the optimizer preferred small effective soil storage, nearly linear runoff generation, and rapid fast-reservoir release. These values should not be interpreted as unique physical measurements. They may compensate for simplified soil properties, a single climate gauge, routing assumptions, and the missing operational behaviour of Shand Dam. The presence of many non-dominated solutions confirms parameter interaction and equifinality.

## Model limitations and uncertainty

The strongest structural uncertainty is reservoir operation. Although the outlet is represented as a standard reservoir, the model uses a generic weir relationship and does not include daily gate settings, target levels, seasonal rule curves, flood-control decisions, or minimum-flow releases. Consequently, a rainfall-runoff model may generate reasonable inflow while still failing to match the measured regulated outflow. This limitation is particularly visible in the low-flow duration curve and in events where observed peaks are delayed, reduced, or released independently of same-day meteorological forcing.

Meteorological uncertainty is also important. One climate station was applied to 18 subbasins across nearly 800 km2. Spatial rainfall gradients, convective storms, snowfall under catch, and elevation differences cannot be fully represented. The HRU structure distinguishes four land-use types but uses one main non-lake soil profile, and it does not explicitly represent tile drainage, wetland connectivity, agricultural management, or urban drainage networks. These limitations place a practical ceiling on the efficiency achievable by parameter calibration alone.

# Answer to the Model Question

## Answer to the project question

The final Raven HBV-EC model reproduced broad seasonal runoff behaviour and several spring and rainfall-driven responses, and multi-objective calibration improved the simulation relative to both the baseline and manual runs. Compared with the baseline, selected PADDS Run 138 increased NSE from 0.2355 to 0.2879 and KGE from 0.5149 to 0.6370, reduced RMSE from 14.154 to 13.661 m3/s, and reduced the magnitude of PBIAS from 26.85% to 10.00%. These simultaneous improvements demonstrate meaningful calibration progress rather than a change that benefited only one diagnostic.

The final diagnostics nevertheless indicate only moderate daily predictive skill. An NSE of 0.2879 means that much of the day-to-day discharge variability remained unexplained, while PBIAS of -9.998% indicates that total simulated discharge was still approximately 10% below the observed volume. The hydrograph provides similar evidence. The largest observed event was approximately 225 m3/s on April 16, 2014; simulated discharge on that date was about 120 m3/s, and the simulated maximum of approximately 164 m3/s occurred three days earlier. The flow-duration curve also showed insufficient sustained flow: observed median discharge was approximately 5.18 m3/s, compared with a simulated median of approximately 1.95 m3/s. Thus, the model represented several runoff-producing periods but did not consistently reproduce peak magnitude, peak timing, or moderate and low flows.

The remaining mismatch is consistent with the difference between the modelled system and the observation. HBV-EC provides a defensible representation of rain-snow partitioning, degree-day melt, soil-moisture-controlled runoff generation, evapotranspiration, fast and slow recession, and catchment routing. A standard reservoir is also included at the outlet, but it is routed using a generic weir relationship. It does not include daily gate settings, target levels, seasonal rule curves, flood-control decisions, or minimum-release requirements. Because WSC 02GA016 is immediately below Shand Dam, the measured hydrograph reflects both watershed inflow and operational decisions. Calibration of soil, snow, and baseflow parameters cannot fully reproduce releases that are deliberately delayed, attenuated, or scheduled independently of same-day meteorological forcing.

Therefore, the model reproduces daily discharge only to a limited but useful extent. It is credible as a conceptual watershed-runoff model and as a framework for examining parameter sensitivity, seasonal water balance, equifinality, and multi-objective calibration. It can indicate broad runoff timing and the direction of parameter effects. It is not sufficiently reliable for predicting exact daily Shand Dam releases, operational low flows, or design flood outflows at 02GA016. The project question is answered by concluding that the HBV-EC structure represents important basin processes, but reliable prediction of regulated downstream discharge requires explicit time-varying reservoir-operation data, along with improved spatial meteorological forcing and soil/drainage representation.

# Conclusions

A complete daily Raven HBV model was developed for the Grand River below Shand Dam. The final model uses 2013-2023 meteorological and streamflow data, 18 connected subbasins, 44 HRUs, four HRU types, HBV snow and runoff processes, triangular catchment routing, and two standard reservoirs. The data, discretization, conceptual structure, and selected algorithms were documented and justified in relation to the basin and project question.

Manual calibration was performed first using six critical parameters. Automatic calibration then used a four-objective PADDS formulation with 200 evaluations. A balanced solution was selected from 24 non-dominated alternatives using a normalized equal-weight distance-to-ideal criterion. The selected model produced NSE = 0.2879, RMSE = 13.6608 m3/s, KGE = 0.6370, and PBIAS = -9.998%. These results improve all four diagnostics relative to the baseline and demonstrate meaningful calibration progress.

The project also demonstrates that improved parameter values cannot overcome a major structural mismatch. WSC 02GA016 measures regulated outflow, while the model includes only simplified standard-reservoir routing. The final model is therefore most credible as a representation of watershed runoff generation and least credible as a predictor of exact regulated daily releases. Explicit dam-operation data, spatial meteorological forcing, and more detailed soil and drainage HRUs are the most important next steps.

# Reference

- Craig, J. R. et al. (2020). Flexible Watershed Simulation with the Raven Hydrological Modelling Framework. Environmental Modelling & Software.

- Raven Development Team. Raven User’s and Developer’s Manual, Version 4.1. University of Waterloo.

- Bergström, S. (1995). The HBV Model. In Computer Models of Watershed Hydrology.

- Han, M. et al. (2023). BasinMaker 3.0: A GIS Toolbox for Distributed Watershed Delineation. Environmental Modelling & Software.

- Matott, L. S. (2017). OSTRICH Optimization Software Toolkit—User’s Guide, Version 17.12.19.

- Asadzadeh, M. and Tolson, B. A. (2013). Pareto Archived Dynamically Dimensioned Search for Multi-objective Optimization. Engineering Optimization.

- Nash, J. E. and Sutcliffe, J. V. (1970). River Flow Forecasting through Conceptual Models. Journal of Hydrology.

- Gupta, H. V. et al. (2009). Decomposition of NSE and Development of the Kling–Gupta Efficiency. Journal of Hydrology.

- Moriasi, D. N. et al. (2007). Model Evaluation Guidelines for Watershed Simulations. Transactions of the ASABE.

- Environment and Climate Change Canada. Historical Climate Data and Water Survey of Canada Streamflow Data for Station 02GA016.

- HydroSHEDS. Hydrological Data and Digital Elevation Model.

- Natural Resources Canada. 2020 Land Cover of Canada Dataset. Government of Canada Open Data Portal.

- Government of Ontario. Ontario Soil Data. Ontario GeoHub.

- Grand River Conservation Authority. Shand Dam and Reservoir Information.